Bulb Manipulation (89c51)

Develop By: Rohan Arora

Project Description

- This project is written for self-practice of embedded C programming.

- In this project, code is written to manipulate an electric bulb with the help of SMS service through mobile.

- An SMS can be sent to switch OFF/ON connected Bulb.

- In this project bulb is used however multiple things can be connected(each at once) to be controlled through mobile SMS service.

Hardware and Software Information
Controller Used: 89c51

- In this project C language is used as programming language.

- 1 Bulb

- This code is tested on 8051 development board. So to execute its better if it is executed on development board. This is because development board has in built ULN Driver(Driver to use relay).

- GSM/GPS board SIM-808 is used for this.

- A mobile sim is required for this.

- UART protocol is used to transfer data.(Hand shake principal is not required as those things are taken care by microcontroller itself).

- Start bits and stop bits are used in frame of UART data including 8 bits of data.

Requirements

Note:

As this is a very simple and small project, there are no separate high and low level requirements. Below requirements are written in such a way that requirements are self-explanatory and coding can be done by reading them.

As this code is just for practicing and also in initial phase, it is totally un-optimized and can be optimized at many places.

- Main will be entry point for program.

- Whole global data shall be initialized here.

- UART initialization is called to initialize UART specific registers.

- GSM initialization is called to initialize GSM module with related commands.

- Below commands shall be used for GSM module.

"AT+CMGF=1\r"           -->To change character encoding sent by GSM module in english.

"AT+CPMS=\"SM\"\r" -->To change used memory to store/send sms to sim.

"AT+CMGD=1,4\r"       -->To delete all the present messages in selected(sim) memory.

"AT+CMGR=1\r"           -->To read first available unread message.

- After initialization processor will go in idle state and wait for any incoming message.

- Once message received, command to read 1st unread message shall be sent.

- Once message read successfully and stored in buffer, sim memory shall be cleared by using command "AT+CMGD=1,4\r".

- From buffer exact message shall be extracted and checked if message is 'H' or 'h'. If message is one of them then bulb shall be switched ON. No action performed otherwise.

- From buffer exact message shall be extracted and checked if message is 'L' or 'l'. If message is one of them then bulb shall be switched OFF. No action performed otherwise.

- After performing these steps, processor shall again go to idle state and start waiting for new message.

Note:

- A buffer is used to store data coming from GSM module, and buffer is nothing but a character array of size 50. Point to be taken care is that counter for buffer has to be reset time to  time, otherwise it can lead to array overflow, since coding is done in C and C does not check for array bounce, Coder is sole responsible for these points.

- Code does not contain any checks related to UART/GSM functionality.

Code and Proteus design

For Code and Proteus design refer below github link:

https://github.com/CreativeGroup8592/GPS-GSM-Vehicle-Tracking-System/trunk/Bulb_Manipulation